[image: image1.png]


[image: image5.jpg]caple.in |

2] Nanxing



[image: image5.jpg]     

Drilling Machine NDB301R
[image: image28.png]



Description

[image: image6.png]



	NDB301R, with 3 vertical drilling rows on bottom and one horizontal row on left side. and one side row of bottom row can overturn to use as horizontal drilling rowEach row are controlled by two motors, and drilling block is able to rotate from 0-90°
Easy operation. Save money save place.

[image: image7.jpg]


[image: image2.png]€

N

£

eececeoscen

N








Details
[image: image8.jpg]



	1. Machine body

· Heavy duty machine body is welded with square tube and laser-cut, then through processing of advanced CNC machine for metal work, hence enhance the stability and strength, reduce the vibration in the process; Machine body have been designed as one organic structure of high rigidity; 

· [image: image9.jpg]


Frame body pass the heat treatment and vibration aging treatment to guarantees the whole service life without deforming.
· After sandblasting and painting, the top-grade CNC metal processing machines are used to process the body and components to make sure perfect quality and detail.


	[image: image10.jpg]


[image: image11.png]


[image: image12.jpg]


[image: image13.png]






	2. Quality control
· Advanced measuring equipment and strict QC system also help to make sure the final machine we offer to our customers are of high quality;
· All the machines are delivered with Quality Certificate signed by QC specialist.
	[image: image14.jpg]


[image: image15.png]


[image: image16.jpg]




	3. Professional assembly line
· Assembly is final and important step for better machine performance. Nanxing’s professional assembly lines from machine body to small electrical components were by experienced technicians that guarantee machine standardization and performance.

	[image: image17.png]




	4. Lengthwise positioning device

· Positioning device with ruler.
	[image: image18.png]




	5. Drilling rows
· Two independent drilling blocks in row, each can rotate to 90°; the minimum distance between each of two rows is 128mm

· Two horizontal rows on both sides and 3 rows on bottom (11*2*3=66), each drill bit’s space is 32mm, conform to 32mm system hole
· Drilling block is 1.5kw 2750rpm, easy to drill hinge hole.
 
	[image: image19.jpg]


[image: image20.png]




	1. Reversible drilling row
· One vertical row is able to turn over to be used as horizontal drilling row by just one click.
	[image: image21.png]




	6. Drill Layout
Spacing of drills: 332mm
Max vertical drilling depth: 50mm

Max horizontal drilling depth: 40mm

Min space of two vertical drills: 128mm

	[image: image3.png]€

N

2
i

N

ASHTTE N K FRE R





	7. Drilling row adjustment 

· Drilling rows position is available at a glance thanks to digital display on each boring row. Easy to adjust bdrilling rows position and drilling depth.


	[image: image22.png]




	8. Top presser
· Top pressers, the height and position are easy to adjust based on different panel size. Fall down automatically in the process to fix the work piece from top to avoid movement during bottom drilling. 
	[image: image23.jpg]




	9. Guiding rail
· Drilling rows move along stronger guiding rail, wearproof of longer service life
	[image: image24.jpg]




	10. Quick coupling of drill bits
· All drill bits are using quick coupling, which make it more convenient to change drill bits. there are also boring depth adjusting device and vertical drill bits feed decelerator device.


	[image: image25.jpg]


[image: image26.png]


[image: image27.png]




	11. Drill bit bracket
· Put all useful drill bits on bracket, fast to access when operator wants to change drill bit.
	

	12. Safety device
· Machine could stop at once while pulling the emergency string; prevent any possible hurt on operator in the process.
	[image: image4.jpg]




	13. Operating Pedal
· An operating pedal is easy to start and stop machine according to requirement.
	

	14. Operation panel
· Button type operation, support synchronous drilling model and non-synchronous mode avoids horizontal drills and vertical drills hit with each other when process cabinet cam fitting’s holes
	

	15. Electrical parts
· Electric components are of high quality and famous brand.
	


Technical Data
	Model
	NDB301R

	Max panel width
	2500mm

	Min. panel width
	90mm

	Max. panel thickness
	70mm

	Spacing of drills
	32mm

	Max vertical boring depth
	70mm

	Max horizontal boring depth
	40mm

	Total power
	8.2kw

	Overall dimension
	3460×2835×1575mm

	Approx. weight
	1520kg


*Thanks for the attention!
The company continuous to improve the product specifications and design details, the specifications are subject to change without notice. 220514
This row is rotatable








Drilling Machine NDB301R                                                             1

